## Stormwater Management Plan Submittal for Minor

## Land Disturbance Projects

Projects with less than 1 acre of land disturbance and are exempt from stormwater design criteria may provide the minimum criteria detailed below.  Exemptions from stormwater design criteria must propose no change to the drainage area, no change to the storm drain system, no major grading changes, and no net increase in built upon area.

Any project with a land disturbance area of 0.1 acre or greater requires a Stormwater Management Plan (SWMP), approved by Environmental Health and Safety (EHS) Stormwater. No land disturbance can begin without the issuance of a SWMP approval letter.

| PROJECT NAME | 202535043: Grading – wolf village | LOD AREA (SF) | 25,500 |
| --- | --- | --- | --- |
| SUBMITTAL DATE | 06.11.2025 | NET INCREASE IN BUILT UPON AREA (SF)* | 0 |
| PROJECT MANAGER | Mike Riha | IS LOD WITHIN THE NEUSE BUFFER ZONE? | No |
| CONSTRUCTION START DATE | 06.16.2025 | APPROXIMATE CONSTRUCTION DURATION | 3-4 weeks |

* net increase in built upon area must be 0 sf or a stormwater control measure (SCM) design is required.

PROJECT DESCRIPTION

The scope of this project is to excavate and level the soil of the courtyard in the center of the Wolf Village residence halls to match the grades of the existing sidewalk.  Excavated soil will be hauled off site. Sod will be installed to cover the disturbed area—no new impervious area is proposed.

SUBMITTAL REQUIREMENTS

- Erosion and Sediment Control Plan - show all applicable E&SC controls

- Stabilization Plan - show final stabilization condition (hardwood mulch, seed, sod, other plantings)

- Standard Details

- Sequence of Construction

- Erosion and Sediment Control Notes and Vegetation Establishment Notes

EROSION AND SEDIMENT CONTROLS FOR PLAN

☒ Limits of Disturbance (LOD)

☒ Tree Protection

☒ Construction Entrance

☒ Inlet Protection

☒ Silt Fence

☐ Stockpile Area(s)

STANDARD DETAILS TO BE ATTACHED

☒ Tree Protection

☒ Construction Entrance

☒ Silt Sock/Wattle for Inlet Protection

☒ Silt Fence

☐ Concrete Washout Detail

☐ Dewatering Device (Pump and Filter Bag) Detail

☒ Temporary Seeding Recommendations

☐ Permanent Seeding Recommendations

☒ NCG-01 Self-Inspection Detail

☒ NCG-01 Recordkeeping and Reporting Detail

☒ NCG-01 Ground Cover Detail

☒ NCG-01 Materials Handling Detail

SEQUENCE OF CONSTRUCTION

1. HOLD A PRECONSTRUCTION MEETING WITH EHS STORMWATER, THE NC STATE PROJECT MANAGER, AND THE CONTRACTOR.

2. INSTALL ALL EROSION AND SEDIMENT CONTROLS PRIOR TO ANY LAND DISTURBANCE.

3. CONDUCT SITE WORK.

4. CONTRACTOR TO PERFORM SELF-INSPECTIONS WEEKLY AND WITHIN 24 HOURS OF RAINFALL EVENTS GREATER THAN 1.0 INCH. SELF-INSPECTION FORMS SHALL BE EMAILED TO EHS STORMWATER.

5. ONCE SITE WORK HAS BEEN COMPLETED, STABILIZE SITE AREA.

6. UPON PERMANENT STABILIZATION OF SITE, HOLD A SITE MEETING WITH EHS STORMWATER, THE NC STATE PROJECT MANAGER, AND THE CONTRACTOR.

7. WITH APPROVAL, REMOVE EROSION AND SEDIMENT CONTROLS. REQUEST SITE CLOSEOUT WITH EHS STORMWATER.

EROSION & SEDIMENT CONTROL NOTES

1. THE CONTRACTOR MUST SCHEDULE A PRECONSTRUCTION MEETING WITH NC STATE, ENVIRONMENTAL HEALTH AND SAFETY, STORMWATER AT LEAST 5 DAYS PRIOR TO BEGINNING ANY LAND DISTURBING ACTIVITIES.

2. A COPY OF THE STORMWATER MANAGEMENT PLAN (SWMP) APPROVAL LETTER AND THE APPROVED PLANS AND PERMITS MUST BE MAINTAINED ON THE SITE AT ALL TIMES.

3. ALL EROSION AND SEDIMENT CONTROL MEASURES ARE TO BE PLACED PRIOR TO CLEARING AND/OR LAND DISTURBANCE.

4. SOIL STOCKPILES MUST BE HAULED IMMEDIATELY OFF SITE OR COVERED AT THE END OF THE DAY.

5. THE CONTRACTOR MUST INSTALL AND MAINTAIN THROUGHOUT THE PROJECT CONSTRUCTION ALL EROSION CONTROL MEASURES SHOWN WITHIN THESE PLANS IN ACCORDANCE WITH APPLICABLE NORTH CAROLINA DEPARTMENT OF ENVIRONMENTAL QUALITY (NCDEQ) AND NC STATE EROSION AND SEDIMENT CONTROL REGULATIONS.

6. SELF-INSPECTIONS ARE REQUIRED AT LEAST ONCE PER 7 CALENDAR DAYS AND WITHIN 24 HOURS OF A RAINFALL EVENT EQUAL TO OR GREATER THAN 1.0 INCH PER 24 HOUR PERIOD. REPORTS MUST BE SENT TO NCSU EROSION AND SEDIMENT CONTROL INSPECTOR. DEMLR MONITORING FORM, INSPECTION AND MONITORING RECORDS FOR ACTIVITIES UNDER STORMWATER GENERAL PERMIT NCGO1 AND SELFINSPECTION RECORDS FOR LAND DISTURBING ACTIVITIES PER G.S. 113A-54.1 OR SIMILAR MAY BE USED.

7. WHERE SEDIMENT IS TRANSPORTED ONTO A PAVED ROAD SURFACE, THE ROAD SURFACE MUST BE CLEANED THOROUGHLY AT THE END OF EACH DAY. SEDIMENT MUST BE REMOVED FROM THE ROADS BY SHOVELING OR SWEEPING AND TRANSPORTED TO A SEDIMENT CONTROL DISPOSAL AREA. STREET WASHING MUST BE ALLOWED ONLY AFTER SEDIMENT IS REMOVED IN THIS MANNER.

8. DURING DEWATERING OPERATIONS, WATER MUST BE PUMPED INTO AN APPROVED FILTERING DEVICE PRIOR TO DISCHARGE TO RECEIVING OUTLET.

9. TURBIDITY IN THE RECEIVING WATER MUST NOT EXCEED 50 NEPHELOMETRIC TURBIDITY UNITS (NTU). ADDITIONAL BEST MANAGEMENT PRACTICES MAY BE REQUIRED BEYOND THOSE SHOWN ON THE PLAN.

10. ANY AREAS OF EXPOSED SOILS THAT WILL NOT BE DISTURBED WITHIN 14 DAYS MUST BE STABILIZED.

VEGETATION ESTABLISHMENT NOTES

1. ALL PERMANENT PERVIOUS AREAS MUST BE TILLED TO A DEPTH OF 12 INCHES IMMEDIATELY PRIOR TO LANDSCAPING ACTIVITIES. THESE AREAS MUST NOT BE SUBJECT TO COMPACTION.

2. ALL SEEDED AREAS MUST BE COVERED IN BIODEGRADABLE EROSION CONTROL MATTING. IF MORE THAN ONE FABRIC WIDTH IS REQUIRED, A MINIMUM OVERLAP OF 6” MUST BE PROVIDED.

3. STRAW BLANKETS MUST BE INSTALLED ON ALL FLAT, SEEDED AREAS. STRAW FIBERS MUST BE DRY AND FREE OF MOLD, WITH THE STRAW FIBERS EVENLY DISTRIBUTED THROUGH THE BLANKET SUCH THAT NO BARE AREAS ARE PRESENT.

4. DRAINAGE DITCHES AND SLOPES UP TO 3:1 GRADE MUST BE PROTECTED WITH AN EROSION CONTROL BLANKET COMPOSED OF 80% WOOD OR CELLULOSE FIBERS. THE FIBERS MUST BE 6-INCHES IN LENGTH AND EVENLY DISTRIBUTED THROUGH THE BLANKET WITH NO BARE AREAS. NETTING MUST HAVE A 1” X 2” GRID.

5. NATURAL FIBER MATTING MUST BE USED IN AREAS WITH HIGH VELOCITY FLOW OR AREAS WITH SLOPES GREATER THAN 3:1 GRADE. THE MATTING MUST HAVE AN OPEN WEAVE THAT IS UNIFORM AND CONSISTENT, WITH A MINIMUM WIDTH OF 6.5’ AND A MINIMUM WEIGHT OF 11.8 OZ PER SQUARE YARD.